xspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspec

xspec								  xspec

xspec		Detector response: Format for XSPEC		  xspec

xspec								  xspec

xspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspec

******Philosophical issues******

How do we define the detector spectral response? All detectors have

spectral information defined by channels. Even detectors with no

spectral resolution (e.g. the Einstein Observatory HRI) can be thought

as a single channel detector. Detected counts are apportioned among the

channels, based on the incident photons energy. However, the

correspondence is not exact, so that a given energy photon has a

probability of being counted in one of several channels.

The incident photon spectrum is divided up into energy ranges. The ith

energy range runs from E(i-1) to E(i), in keV. The response "matrix" is

defined by the values R(j,i) = to the average PROBABILITY that an incident

photon in the ith energy range will be detected in the jth channel. By

this definition, the response does not include any detector area term. The

actual values of the energy ranges, the E(i), are chosen by the generator

of the response matrix. They should be chosen small, compared to the

detector intrinsic resolution, but not too small, as the computational

overhead in evaluating the spectral models is roughly proportional to the

number of energy ranges. The ranges need not be evenly spaced, and because

the model evaluation is supposed to use the R(j,i) as the AVERAGE response

over the energy bin, it might prove more accurate to use the edges in the

detector response as some of the bounds.

For convenience the other bounds should be spaced by submultiples of

2, i.e. spacings of 2 keV, 1 keV, 0.25 keV, 0.03125 keV, etc. This is

not a strict requirement, however this makes it easier to combine fits

of multiple detectors with differing amounts of resolution.

The definition of the R(j,i) is such that it should include ALL

the spectrally dependent information, including telescope vignetting,

correction for scattering outside the integration pixel, etc. The

quantity of R(j,i) summed over all the PHA channels (summed over j) is

the probability that an incident photon in the ith range was detected at

all, i.e. the quantum efficiency of the experiment in the ith channel.

If S(i) is the incident number of photons cm**-2 s**-1 and A is the total

effective area of the instrument, independent of spectral corrections,

then

C = 0.

do I=1,number_energy_ranges

C=S(i)*R(j,i)

end do

C=C*A

is the expected count rate (counts per second) in the jth PHA channel.

***** RSP file format (SF) ******

Each data file has associated with it a response file to convert

incident model spectra to the expected number of counts.  This response

file contains all the energy dependent information about the detector

response.  The response files (default extension .RSP) are SF format

files (see the associated documentation) with file type `XSPEC rspnse'.

The `SF history' package should contain ASCII records with all the

information about what went into the creation of the response (e.g.

creation date, special fudge factors, etc.). After that there are a

variety of packages, which should be in the following order:

'detect info ' - describes the detector

optional detector dependent packages, not used by XSPEC

optional method packages, containing numerical information

used in creating the response, not used by XSPEC

'grouping    ' - A grouping card, if the response has been

compressed. (NB. the current release of XSPEC

does NOT support compressed responses)

(see DATA.DOC for a description of the grouping

card).

'chanel bound' - An approximate representation of the energy

range that a single channel corresponds to.

creating the response, not used by XSPEC

'response    ' - The actual information for the response.

Additional packages can be inserted in any order.  They will be ignored

by XSPEC.

The 'detect info ' package has a 100 byte header record organized as

follows:

DETID	c16	- A character string containing the detector ID

NBIN	i4	- The number of entries after compression

NCHAN	i4	- The number of PHA channels for the detector

before grouping (if NBIN is not equal to NCHAN

then there must be a grouping package)

RSPAREA	r4	- The total effective area of the RESPONSE. N.B.

In the discussion above, the area is to be

considered the response to a flux of photons

per cm**2. Thus this number should be 1.0. Other

systems might scale the response differently,

in which case the number here will be non unity.

Effectivly, the response is for a fictitious

detector with RSPAREA as its area.  This is NOT

the same number as the AREA of any PHA file.

NRANGE	i4	- The number of (contiguous) energy ranges over

which there is significant response.

E0	r4	- The lower limit of the first energy range.

SPARE	44	- Reserved for future expansion (should be set to 0)

The 'chanel bound' package is NBIN*2 values which give the

approximate boundaries in keV that the channel corresponds to.  All the

information is included in a header which is dimensioned real*4

EBOUND(2,NBIN) where EBOUND(1,I) is the lower limit of the Ith bin and

EBOUND(2,I) is the upper limit.  Note that due to the inexact nature of

X-ray detectors, these values are only illustrative of where the photons

that produce the observed counts may be coming from.

The `response' package contains all the R(i,j) values.  Because

this matrix can be very sparse, only the significant values are stored.

The package has NRANGE*3 subsidiary records, of variable length, a set

of three for each energy range.  The first contains a record of

information for that range of 12 bytes:

E	r4	- The upper energy of the current energy range.

(The lower energy is the value of E in the

previous record, or E0 from the `detect info'

package for the first record).

NGROUP	i4	- The total number of groups of contiguous bins

with response.

NENTRY	i4	- The total number of R(i,j) values written on

this record. The second record consists of:

BINRNG	i4(2,NGROUP)- The lower and upper bin numbers for each

group. The third and final record for the

set is

INRESP	i4(NENTRY)  - The actual R(i,j) values. Note that if an

energy range has no non-zero response then

both NGROUP and NENTRY should be zero. Also

NENTRY is equal to the sum over all the

groups of BINRNG(2,i) - BINRNG(1,i) + 1.

However, even if NGROUP and NENTRY are zero,

three records are still in the package. The

values of the second and third should be

ignored in this case.