xspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspec

xspec                                                       xspec

xspec   Detector specific information in PHA/RSP files      xspec

xspec                                                       xspec

xspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspecxspec

Such information is contained in an SF package that follows the

`file info' package in PHA files and the `detect info' package in RSP

files. The package name is unique for a given detector.  All EXOSAT

detectors have "EXOSAT" as the first six characters.

ME detectors

For ME detectors the package name is `EXOSAT ME'.  The package

consists of a 108 byte header and 1 to 16 100 byte auxiliary records.

The header is:

GAS	c2	"AR" if Argon, "XE" if Xenon and "AX" if both

Argon and Xenon data are included.

INCRES	L1	If true, then resolution information is

contained in the aux. records.

INCGN	L1	If true, the corrected gain information is

included in the aux. records.

REVIS	i4	A flag indicating the revision level (SHF key)

of the information used to produce this file

(e.g. detector areas, orientations, FWHM, or

spectral calibration info if this package is

found in an RSP file).

SHF	i4	SHF key of observation date.

H1ANG	r4	Offset angle of the first half (average if

the data includes multiple offsets).

H2ANG	r4	Offset angle of the second half.

INCDET	L1(8)	If true, then the Ith detector is included

in the accumulation.

SYSTEM	i4	Coordinate system (see 'source info' package)

SCXAXIS	r8(3)	Space craft X-axis

SCYAXIS	r8(3)	Space craft Y-axis

SPARE	20	Future expansion.

Each included detector will have one auxiliary record (two if both

Argon and Xenon data are included):

DETNUM	i4	The detector number (from 1-8 for AR, 9-16 XE).

COLRSP	r4	The collimator response.

ASSAR	r4	The assumed on-axis area.  Thus the contri-

bution of this detector to the area in the

`obs info' package is ASSAR*COLRSP.  During the

life of the instrument this ASSUMED number

should probably remain constant.  If updates

are found in the process of the calibration

they should be included in the program calcula-

ting the response file (as they are probably

small).  (When this package is found in an

RSP file it is the value used in scaling the

rsponse.  If the current cal knows that the

area is TRAREA, then all the responses have

been multiplied by an

additional correction factor TRAREA/ASSAR.)

AG	r4	The analogue gain setting.

GCOEF	r4(4)	Gain coefficients.

RESOL	r4(8)	Resolution coefficients.

USEDRSPAREA r4

DETINTT r4	integration time

SPARE	28	Future expansion

GSPC detector

The package `EXOSAT GSPC' describes the GS status. It has a

120 byte header.

BLWSTART	i2	Burstlength window start

BLWEND		i2	Burstlength window end

NOMINALGAIN	r4	Nominal gain setting

GAINCOR		r4	Gain correction factor

USELINE		l1(4)	Flag for using up to 4 lines to

calculate gain correction factor,

currently not possible, set to false

USEGAINCOR	l1	Flag for using GAINCOR

LINECHAN	i4(4)	Channel numbers of up to 4 lines

currently set to 0

REVIS, SHF, SYSTEM, SCXAXIS, SCYAXIS, COLRSP, ASSAR,

and USEDRSPAREA		as for ME

flgcolrsp	i2	Flag for collimator response being

in pha file

SPARE		13	Future expansion

LE detectors

The package 'EXOSAT LE' has a 120 byte header.

LEID		i2	Telescope id (1 for LE1, 2 for LE2)

DET		c3	CMA or PSD

FILTER		i2	Filter number 	2: ppl

3: 4000 LEX

6: alp

7: 3000 LEX

8: bor

REVIS, SHF, SYSTEM, SCXAXIS, SCYAXIS as for ME and GSPC

NOPIX		i2	Number of pixels in x

NOPIY		i2	Number of pixels in y

PIXSIZE		i2	Pixel size in x (arcsec)

PIYSIZE		i2	Pixel size in y (arcsec)

XPIX		r4	Source centroid pixels (abscissa)

YXPIX		r4	Source centroid pixels (ordinate)

STTIMEYY, STTIMEDDD, STTIMEHH, STTIMEMM, STTIMESS

i2	Start time in year,day,hour,min,sec

ENDTIMEYY, ENDTIMEDDD, ENDTIMEHH, ENDTIMEMM, ENDTIMESS

i2	End time in year,day,hour,min,sec

BGBOXNO		i2	No of background boxes

SPARE		17	Future expansion